# Allen atlas registration tools Freiburg

## System requirements

Matlab 2012b incl. Symbolic Math Toolbox maybe: Curve Fitting Toolbox, Image Processing Toolbox, Signal Processing Toolbox, Statistics Toolbox, Optimization Toolbox, Parallel Computing Toolbox

## Add relevant paths

- All scripts are located under /Volumes/VCI/matlabToolsFreiburg/

- Add paths in following order

  - spm8

  - matlab_new

  - allen, AMA, Fiberviewer3D (no subfolders necessary)

- Important: remove all other SPM versions from matlab path

## Description of main toolboxes

- Access to main GUIs: BrAt_StartGui

- AMA (Automatic Mouse Analyser, Datenimport): AMA_gui2

- Visualization: fiberViewer3D; für fMRI: StartGui_fv3d

## Import Bruker dataset (AMA)

- In case data import fails, remove AMA-unknown scans first

- AMA_gui2

- Choose Folder: folder containing raw data

- Start AMA: new folder p_data_automatic is generated

- Optional: crop/repair to reduce processing time by reducing image volume

- Save Pathfile: p_data_automatic/pathfile-sorted_folders.txt is generated

## Image alignment (AMA)

  - Load SPM Mouse

  - Load Paths  p_data_automatic/pathfile-sorted_folders.txt

  - Extract t2  select all, two files are created per subject (t2_1.nii and t2_safe which will not be modified in further processing steps)

  - Caution: coregistration t2 is meant for T2B0 matching

  - SegmentationSelect TPM: "New" copies SPM mouse TPMs in local p_data_automatic/template directory

    - Caution: don't click on "Check template to t2 registration"

    - R. click on TPM, reorient imagecurrent image  opens menu to rigid transform image to TPM

    - Several files (c1, c2, c3) are generated under results

    - Typical parameter settings

    - "Apply to images" in rigid transform menu

    - Transform all three TPMs, save matrix for future reference "yes", name "reorient.mat", store in p_data_automatic/template

    - Check template to t2 registration "OK"

  - Check image alignment

    - Load paths

    - Deformationselect mouset2 several "wc…" niftis are generate

    - Checkreg under Load SPM Mouse

  - Repeat procedure for misregistrations

    - Load pathsSegmentationselect misregistered subjects, old TPMs are stored under p_data_automatic/template/tpm_# where # is ascending integer, newest TPM is always under p_data_automatic/template/

    - repeat previous steps

  - Final registration results are stored in y_forward and y_inverse (concatenation of manual and automatic registration)

## Transform other images into Allen space (AMA/SPM)

- y_forward.nii and y_inverse.nii contain the full information and can be used together with SPM's "Deformations" function, sketch attached

## Measure parameter maps in Allen space (BrAt_StartGui)

- Allen atlas: matlabToolsFreiburg/allen/ANO.nii, warped to SPM TPMs wANO.nii

- Allen template: matlabToolsFreiburg/allen/AVGT.nii

- Input: wmasks (weighted)

  - Will be converted to mask with threshold 0.3 (everything above 0.3)

- List nii components (top left)

- Documentation about overlap under AMIR Allen BrAt Readme.pdf